Conceptual Site Model of Contamination			Appendix IV

Site; Railway Cottage, Counter Drain Drove, Tongue End, Spalding.

| Source | Pathway | Receptor | Estimated Risk |
| --- | --- | --- | --- |
| Soils impacted by petroleum hydrocarbons, heavy metals. Asbestos fibres and fragments – confirmed none present. | Direct or dermal contact with soil | On site workers during demolition/ remedial works, excavation and/or development. Site visitors during development. Ecology Buildings and infrastructure. Local offsite tenants and neighbours and future occupiers Future site occupiers. | Moderate – potential for direct contact with contaminated soils Low – Existing vegetation within site and site perimeter appears to be healthy. Low – soil conditions do not appear aggressive to building materials. Low – Hard landscaped area or where soils are capped. Low |

Conceptual Site Model of Contamination – continued

| Source | Pathway | Receptor | Estimated Risk |
| --- | --- | --- | --- |
| Groundwater impacted by heavy metals and Petro Hydrocarbons, PTE’s | Contact with water, vertical and horizontal migration through to surface waters, adjacent ditch & Counter drain. | On site workers during demolition/ remedial works, excavation and/or development. Ecological systems Buildings and Infrastructure. Local off-site tenants. Future site occupiers | Low – minimal, works undertaken above water table. Low Low – foundations are to be above water table. Low – minimal groundwater impact and depth to groundwater. Low – minimal groundwater impact. |