Appendix VI – Results Summary

Soil Test results from WAC test summarised and compared to Soil Guideline Values (SGV’s)

| Determinand | Report No 47973-21 | Report No 47974-21 | SGV’s |
| --- | --- | --- | --- |
|  | Sample No ENV 59806 | Sample No ENV 59807 |  |
|  | Sample 1 | Sample 2 |  |
| pH | 8.3 | 8.7 |  |
| Copper | 0.07 | 0.10 | 200 |
| Zinc | <0.5 | <0.5 | 300 |
| Lead | <0.05 | <0.05 | 300 |
| Arsenic | <0.05 | <0.05 | 37 |
| Cadmium | <0.01 | <0.01 | 22 |
| Nickel | <0.05 | <0.05 | 130 |
| Chromium | <0.05 | <0.05 | 130 |
| Mercury | <0.005 | <0.005 | 10 |
| Selenium | <0.01 | <0.01 | 350 |
| Molybdenum | <0.05 | <0.05 | 4 |
| Fluoride | 4 | 2 | 500 |
| EPH (C10-C40) | 397 | 736* | 500 |
| Total Organic Carbon | 10.1 | 6.6 |  |
| Organic Matter (LOI) | 10.8 | 6 |  |

Notes on results;

- SGV guideline level comparisons are taken from the CLEA model & Environmental Permitting Regulations and other linked legislation.

- All results are expressed in mg/kg unless otherwise stated and represent the maximum permissible in soil.

- The pH level in both samples are high at 8.3 to 8.7.

- At low pH levels, PTE’s become more available to plant uptake therefore to a large extent, pH will govern permitted levels.

- EPH includes category 1 & Category 2 products.

- EPH levels exceeded on sample 2. Mitigation measures include importation of clean ‘fill’ material to cover existing by 300mm+

- Assumed land use is ‘Residential use with plant uptake.’