Method Statement for the demolition of buildings at Spalding Road, Pinchbeck

- All services to be disconnected prior to demolition commencing.

- Site to be fenced using 2m high block and mesh type fencing prior to work commencing.

- Building to be soft striped by hand, removing all loose furniture, fixtures and fittings etc.

- All arising materials to be loaded onto trucks for removal off of site.

- Asbestos/cement roof sheets to be carefully removed by hand, by men working from M.E.W.P’s and all arising materials to be placed into skips for disposal off of site.

- 360 JCB excavator to demolish remaining structures, including floor slab and foundations where necessary

- All arising materials to be loaded onto lorries for recycling off of site.

- Site to be left in a clean, tidy and safe condition at the end of each shift and upon completion of works.

- All relevant PPE and RPE to be worn at all times where necessary.

- All work to be carried out following all current HSE guidelines.

- All plant on site to hold up to date test certificates and to only be operated by competent operators holding up to date CPCS card or equivalent.